# Perf commands for Verification - DEAE 2597

- perf record for data page size

  - sudo perf record --data-page-size -e cycles -a -- sleep 5

Without Patches:

Error: unknown option `data-page-size'

Usage: perf record [<options>] [<command>]

or: perf record [<options>] -- <command> [<options>]

With Patches:

[ perf record: Woken up 1 times to write data ]

[ perf record: Captured and wrote 0.822 MB perf.data (2610 samples) ]

[amd@volcano9b0c-os ~]$ sudo perf report --dump | grep -i "page size" | head -n 5

.. data page size: N/A

.. data page size: N/A

.. data page size: N/A

.. data page size: N/A

.. data page size: N/A

- perf record for code page size

  - perf record --code-page-size -e cycles -a -- sleep 5

Without Patches:

Error: unknown option `code-page-size'

Usage: perf record [<options>] [<command>]

or: perf record [<options>] -- <command> [<options>]

With Patches:

[ perf record: Woken up 1 times to write data ]

[ perf record: Captured and wrote 0.822 MB perf.data (2610 samples) ]

sudo perf report --dump | grep -i "page size" | head -n 5

.. code page size: 2M

.. code page size: 2M

.. code page size: 2M

.. code page size: 2M

.. code page size: 2M

- IBS op PMU

  - sudo perf record -e ibs_op// -c 100000 -a

Without Patches:

Initial error:

event syntax error: 'ibs_op//'

\___ Cannot find PMU `ibs_op'. Missing kernel support?

Run 'perf list' for a list of valid events

Usage: perf record [<options>] [<command>]

or: perf record [<options>] -- <command> [<options>]

-e, --event <event>   event selector. use 'perf list' to list available events

With Patches:

[ perf record: Woken up 1 times to write data ]

[ perf record: Captured and wrote 0.743 MB perf.data (3102 samples) ]

  - perf record -e ibs_op/cnt_ctl=1/ -c 100000 -p 1234

Without Patches:

WARNING: Hw internally resets sampling period when L3 Miss Filtering is enabled

Error:

Invalid event (ibs_op/cnt_ctl=1/) in per-thread mode, enable system wide with '-a'

With Patches:

[ perf record: Woken up 1 times to write data ]

[ perf record: Captured and wrote 0.743 MB perf.data (3102 samples) ]

  - perf record -e ibs_op/cnt_ctl=1/ -c 100000 -- ls

Without Patches:

WARNING: Hw internally resets sampling period when L3 Miss Filtering is enabled

Error:

Invalid event (ibs_op/cnt_ctl=1/) in per-thread mode, enable system wide with '-a'

With Patches:

[ perf record: Woken up 1 times to write data ]

[ perf record: Captured and wrote 0.053 MB perf.data (1121 samples) ]

- Perf IBS L3 miss

  - sudo perf record -c 1000 -e ibs_op/l3missonly=1/ --raw-samples sleep 1

(System-wide profile, cycles event, sampling period: 100000)

Without Patches:

WARNING: Hw internally resets sampling period when L3 Miss Filtering is enabled

and tagged operation does not cause L3 Miss. This causes sampling period skew.

Error:

Invalid event (ibs_op/l3missonly=1/) in per-thread mode, enable system wide with '-a'.

With Patches:

WARNING: Hw internally resets sampling period when L3 Miss Filtering is enabled and

tagged operation does not cause L3 Miss. This causes sampling period skew.

[ perf record: Woken up 1 times to write data ]

[ perf record: Captured and wrote 0.022 MB perf.data (50 samples) ]

  - sudo perf record -e ibs_op/cnt_ctl=1,l3missonly=1/ -c 100000 -a

(System-wide profile, cycles event, sampling period: 100000)

Without Patches:

WARNING: Hw internally resets sampling period when L3 Miss Filtering is enabled

and tagged operation does not cause L3 Miss. This causes sampling period skew.

Error:

The sys_perf_event_open() syscall returned with 22 (Invalid argument) for event (ibs_op/l3missonly=1/).

/bin/dmesg | grep -i perf may provide additional information.

- IBS fetch PMU

  - perf record -e ibs_fetch// -c 100000 -a

(System-wide profile, fetch ops event, sampling period: 100000)

Without Patches:

Error:

Invalid event (ibs_op/l3missonly=1/) in per-thread mode, enable system wide with '-a'.

vent syntax error: 'ibs_fetch//'

\___ Bad event or PMU

Unable to find PMU or event on a PMU of 'ibs_fetch'

Initial error:

event syntax error: 'ibs_fetch//'

\___ Cannot find PMU `ibs_fetch'. Missing kernel support?

Run 'perf list' for a list of valid events

With Patches:

[ perf record: Woken up 32 times to write data ]

[ perf record: Captured and wrote 13.988 MB perf.data (279560 samples) ]

  - perf record -e ibs_fetch/rand_en=1/ -c 100000 -a

(System-wide profile, fetch ops event, sampling period: 100000, Random enable)

Without Patches:

Error:

Invalid event (ibs_op/l3missonly=1/) in per-thread mode, enable system wide with '-a'.

vent syntax error: 'ibs_fetch//'

\___ Bad event or PMU

Unable to find PMU or event on a PMU of 'ibs_fetch'

Initial error:

event syntax error: 'ibs_fetch//'

\___ Cannot find PMU `ibs_fetch'. Missing kernel support?

Run 'perf list' for a list of valid events

With Patches:

[ perf record: Woken up 1 times to write data ]

[ perf record: Captured and wrote 1.000 MB perf.data (5987 samples) ]

- LBR v2 patch verification

  - sudo dmesg | grep LBR

[    0.311127] Performance Events: Fam17h+ 16-deep LBR, core perfctr, AMD PMU driver.

[    7.990359] kvm_amd: LBR virtualization supported

  - perf record -b -e cycles:u sleep 10

Without patches:

Error:

cycles:u: PMU Hardware or event type doesn't support branch stack sampling.

With patches:

# To display the perf.data header info, please use --header/--header-only options.

#

#

# Total Lost Samples: 0

#

# Samples: 1K of event 'cycles:u'

# Event count (approx.): 1456

#

# Overhead  Command  Source Shared Object  Source Symbol                 Target Symbol           >

# ........  .......  ....................  ............................  ........................>

#

2.40%  ps       libc.so.6             [.] _IO_sputbackc             [.] _IO_sputbackc       >

1.24%  ps       libc.so.6             [.] _IO_fwrite                [.] _IO_fwrite          >

1.17%  ps       libproc2.so.0.0.2     [.] 0x000000000000fa73        [.] 0x000000000000fa10  >

1.10%  ps       libc.so.6             [.] 0x00000000000447dd        [.] 0x00000000000447f7  >

1.10%  ps       libc.so.6             [.] 0x0000000000058c44        [.] 0x0000000000058c10  >

1.03%  ps       libproc2.so.0.0.2     [.] 0x000000000000fa2d        [.] 0x000000000000faa0  >

1.03%  ps       libproc2.so.0.0.2     [.] 0x000000000000fabf        [.] 0x000000000000fa6e  >

1.03%  ps       libc.so.6             [.] 0x0000000000044801        [.] 0x00000000000447b0  >

1.03%  ps       libc.so.6             [.] 0x000000000016668b        [.] 0x00000000001666d0  >

0.96%  ps       libc.so.6             [.] 0x000000000005888d        [.] 0x00000000000587f2  >

0.89%  ps       libc.so.6             [.] 0x00000000000447b3        [.] 0x00000000000447ba  >

0.82%  ps       libc.so.6             [.] _IO_file_xsputn           [.] _IO_file_xsputn     >

0.76%  ps       libc.so.6             [.] 0x0000000000047380        [.] 0x0000000000047388  >

- LBR with Detailed branch stack analysis:

  - perf script -F +brstackinsn,+brstacksym

Without patch:

script -F +brstackinsn,+brstacksym

Display of branch stack assembler requested, but non all-branch filter set

Hint: run 'perf record -b ...'

With patch:

ps 1889158 498629.243837:          1 cycles:u:  ffffffff87800bf0 [unknown] ([unknown])

ps 1889158 498629.243867:          1 cycles:u:      7f049f861243 [unknown] (/usr/lib64/ld-lin>

ps 1889158 498629.243876:          1 cycles:u:  ffffffff87800bf0 [unknown] ([unknown])

ps 1889158 498629.243886:          2 cycles:u:  ffffffff87800bf0 [unknown] ([unknown])

ps 1889158 498629.243896:          9 cycles:u:      7f049f861e9b [unknown] (/usr/lib64/ld-lin>

ps 1889158 498629.243904:         36 cycles:u:      7f049f861e9e [unknown] (/usr/lib64/ld-lin>

ps 1889158 498629.243912:        142 cycles:u:      7f049f861ea2 [unknown] (/usr/lib64/ld-lin>

ps 1889158 498629.243926:        662 cycles:u:      7f049f86213c [unknown] (/usr/lib64/ld-lin>

00007f049f861e6e        insn: 76 e9                     # PRED

00007f049f861e59        insn: 48 89 14 c6

00007f049f861e5d        insn: 48 8b 42 10

00007f049f861e61        insn: 48 83 c2 10

00007f049f861e65        insn: 48 85 c0

00007f049f861e68        insn: 74 4e

00007f049f861e6a        insn: 48 83 f8 25

00007f049f861e6e        insn: 76 e9                     # PRED

00007f049f861e59        insn: 48 89 14 c6

00007f049f861e5d        insn: 48 8b 42 10

00007f049f861e61        insn: 48 83 c2 10

- Unit tests for Perf Vendor events

- To verify vendor event patches from updated perf binary, grep for the new event from perf list like below.

perf list | grep ls_not_halted_cyc

ls_not_halted_cyc

- Zen 4 Uncore events vendor patches verification.

  - sudo perf stat -e l3_lookup_state.l3_miss

without patches:

event syntax error: 'l3_lookup_state.l3_miss'

\___ Bad event name

Unable to find event on a PMU of 'l3_lookup_state.l3_miss'

Run 'perf list' for a list of valid events

Usage: perf stat [<options>] [<command>]

-e, --event <event>   event selector. use 'perf list' to list available events

with perf patches:

Performance counter stats for 'system wide':

37,033,591      l3_lookup_state.l3_miss

491.751102981 seconds time elapsed

- Zen 4 Core event vendor patches patch verification

  - sudo perf stat -e bp_l1_tlb_miss_l2_tlb_hit sleep 1

without patch:

event syntax error: 'bp_l1_tlb_miss_l2_tlb_hit'

\___ Bad event name

Unable to find event on a PMU of 'bp_l1_tlb_miss_l2_tlb_hit'

Run 'perf list' for a list of valid events

Usage: perf stat [<options>] [<command>]

-e, --event <event>   event selector. use 'perf list' to list available events

with patch:

Performance counter stats for 'sleep 1':

0      bp_l1_tlb_miss_l2_tlb_hit:u

1.001173665 seconds time elapsed

0.001009000 seconds user

0.000000000 seconds sys

- Zen 4 metrics vendor patches verification

  - sudo perf stat -e ls_not_halted_cyc ./test_dispatch_slots

without perf patches:

event syntax error: 'ls_not_halted_cyc'

\___ Bad event name

Unable to find event on a PMU of 'ls_not_halted_cyc'

Run 'perf list' for a list of valid events

Usage: perf stat [<options>] [<command>]

-e, --event <event>   event selector. use 'perf list' to list available events

with perf patches:

Performance counter stats for './test_dispatch_slots':

147,532,695      ls_not_halted_cyc

0.046021315 seconds time elapsed

0.044920000 seconds user

0.000999000 seconds sys

- perf sort: Add sort option for data page size

  - sudo perf mem record -a sleep 1

sudo perf report --stdio --mem-mode --sort=comm,symbol,phys_daddr,data_page_size

without patches:

Usage: perf mem [<options>] {record|report}

-C, --cpu <cpu>       list of cpus to profile

-D, --dump-raw-samples

dump raw samples in ASCII

-f, --force           don't complain, do it

-i, --input <file>    input file name

-p, --phys-data       Record/Report sample physical addresses

-t, --type <type>     memory operations(load,store) Default load,store

-U, --hide-unresolved

Only display entries resolved to a symbol

-x, --field-separator <separator>

separator for columns, no spaces will be added between columns '.' is reserved.

With patches:

# PID, TID, IP, ADDR, PHYS ADDR, DATA PAGE SIZE, LOCAL WEIGHT, DSRC, SYMBOL

10718 10718 0xffffffff90810ae3 0x0160 0x0000000000000000 N/A     0 0x1e05080021 /proc/kcore:perf_ibs_start

10718 10718 0xffffffff90810169 0x0 0x0000000000000000 N/A 0 0x1e05080021 /proc/kcore:perf_ibs_handle_irq

10718 10718 0xffffffff90810169 0x0 0x0000000000000000 N/A 0 0x1e05080021 /proc/kcore:perf_ibs_handle_irq

10718 10718 0xffffffff91519e60 0x0 0x0000000000000000 N/A 0 0x1e05080021 /proc/kcore:local_clock_noinstr

10718 10718 0xffffffff9160170d 0x0 0x0000000000000000 N/A 0 0x1e05080021 /proc/kcore:nmi_restore

10718 10718 0xffffffff90c6e1d7 0xffa00000265ebee8 0x0000000000000000 N/A 0 0x229080144 /proc/kcore:do_vfs_ioctl

17 17 0xffffffff90956ebc 0x0 0x0000000000000000 N/A 0 0x1e05080021 /proc/kcore:cpu_util.constprop.0

10718 10718 0xffffffff90810ae3 0x0 0x0000000000000000 N/A 0 0x1e05080021 /proc/kcore:perf_ibs_start

10718 10718 0xffffffff90810169 0x0 0x0000000000000000 N/A 0 0x1e05080021 /proc/kcore:perf_ibs_handle_irq

10718 10718 0xffffffff90810169 0x0 0x0000000000000000 N/A 0 0x1e05080021 /proc/kcore:perf_ibs_handle_irq

10718 10718 0xffffffff91704104 0x0 0x0000000000000000 N/A 0 0x1e05080021 /proc/kcore:srso_alias_safe_ret

10718 10718 0xffffffff91601717 0x0 0x0000000000000000 N/A 0 0x1e05080021 /proc/kcore:nmi_restore

10718 10718 0x7ff1b7113250 0x0 0x0000000000000000 N/A 0 0x1e05080021 /usr/lib64/libc.so.6:sched_setaffinity